The 9th International Workshop on ADVANCED MATERIALS SCIENCE AND NANOTECHNOLOGY

Ninh Binh City, Vietnam, 7-11 November 2018

Contact Address: IWAMSN 2018 Workshop Secretariat and Administration Staff

Institute of Materials Science, Vietnam Academy of Science and Technology,

18 Hoang Quoc Viet Road, Cau Giay District, Hanoi, Vietnam.

- Phone: (84-4) 3.7568870                              - Fax: (84-4) 3.8360705

- Email: iwamsn2018@ims.vast.ac.vn             - Website: http://www.iwamsn.ac.vn

## ABSTRACT FORM

(Submission Deadline: 5 September 2018)

Title: Resonator for magnetic sensor applications based on the magnetoelectric thin film integrated on Si.

Category                                Oral presentation                    Poster presentation

Session: 2.  Nanostructured Materials and Devices (code: NMD)

Author(s):	*Nguyen T.N.1*, Agnus G.2, Matzen S.2, Maroutian T.2, Giang DTH3 Lecoeur P.2

Institution:

1 Department of Advanced Materials Science and Nanotechnology, University of Science and Technology of Hanoi, Vietnam Academy of Science and Technology, 18 Hoang Quoc Viet, Hanoi, Vietnam

2 Centre de Nanosciences et de Nanotechnologies, CNRS UMR 9001, Univ. Paris-Sud, Université Paris-Saclay, C2N - Orsay, 91405 Orsay, France

3 University of Engineering and Technology, Vietnam National University, Hanoi

Speaker:	Thi-Ngoc Nguyen

E-mail:	ngoc-thi.nguyen@u-psud.fr

Abstract:	Magnetoelectric (ME)-based sensors have been demonstrated as a promising alternative for the magnetic field  detection with high sensitivity on the order of a few tens of nanoteslas [1,2]. To date, most applications focus on laminated composites of using bulk piezoelectric (PZT) materials,  which is a critical limit for miniaturizinginto micrometer-sized devices.This work shows the first attempts of a resonator based on multi-ferroic PZT/TbFeCo multilayers, where magnetic signal was very sensed by the shift of the resonant frequency. In this device, TbFeCo layer acts as a micro-actuator, while the PZT thin film as a micro-sensor. In magnetic  field range less than 0.2 T, the sensor’s sensitivity as high as 250 Hz/T and 487 Hz/T  were achieved  in vacuum environments of … and 5 mbar, respectively). A smaller size and lower power consumption provide a significant advantages in terms of  micro-ME-sensors for sensing magnetic field.

## References:

[1]	M. Li, A. Matyushov, C. Dong, H. Chen, H. Lin, T. Nan, Z. Qian, M. Rinaldi, Y. Lin, and N. X. Sun, “Ultra-sensitive NEMS magnetoelectric sensor for picotesla DC magnetic field detection,” Appl. Phys. Lett., vol. 110, p. 143510, 2017.

[2]	S. Dong, J. F. Li, and D. Viehland, “Ultrahigh magnetic field sensitivity in laminates of TERFENOL-D and Pb(Mg1/3Nb2/3)O3-PbTiO3 crystals,” Appl. Phys. Lett., vol. 83, no. 11, pp. 2265–2267, 2003.